DAHV-WF

DOUBLE DEFLECTION SUPPLY GRILLE WITH WIDE FLANGE - HORIZONTAL FRONT

MODEL DESCRIPTION: Wide flange for additional mounting surface coverage; horizontal-front double-deflection supply grille.

TYPE: Supply grille.

BLADE ARRANGEMENT: Two sets of individually adjustable, semi-airfoil blades spaced on 2/3 in. centers; horizontal front blades and vertical rear blades.

DAMPER: None.

FRAME: Wide-flange architectural aluminum frame.

FINISHES: W - White; S - Silver; M - Mill

PERFORMANCE BASIS: ANSI/ASHRAE Standard 70-2023

SUGGESTED SPECIFICATION

Furnish and install Dayus Model DAHV-WF Double Deflection Supply Grilles of the type, finish, listed size/opening size, and mounting method shown on the plans and air distribution schedules. Units shall include a wide flange for additional mounting surface coverage. The grilles shall have two sets of individually adjustable, semi-airfoil blades spaced on 2/3 in. centers, with horizontal front blades and vertical rear blades for two-plane air-direction control. The frame shall be architectural aluminum with roll-formed aluminum blades, pressure-fitted for rattle-free performance, and supplied with a sealing gasket to help prevent streaking on walls or ceilings. Units shall be furnished in Dayus W White, S Silver, or M Mill finish as scheduled.

(Optional) No screw-hole fastening and all-aluminum construction shall be provided where scheduled. Where a damper is required, provide Dayus Model DAHVD-WF.

The manufacturer shall provide published performance data for the grille or register, which shall be tested in accordance with ANSI/ASHRAE Standard 70-2023.

STANDARD FEATURES

- Wide flange for additional mounting surface coverage

- Architectural aluminum construction with extruded frame and roll-formed blades

- Pressure-fitted blades for rattle-free performance

- Sealing gasket to help prevent streaking on walls or ceilings

- Available on 2 in. standard dimensions; custom sizes manufactured up to 1/16 in. increments.

RELATED OPTIONS / MODEL VARIANTS

- DAHVD-WF - Opposed blade damper

- DAHV-WF-TB - No screw holes

- DAHV-WF-AA - All aluminum construction

- DAVH-WF - Vertical front version

Note: Use bracketed finish, size, mounting, and option selections from the project plans and air distribution schedule.