DALTL-30-WF

30° THINLINE LOUVER WITH WIDE FLANGE

TYPE: 30° thinline louver with wide flange frame.

BLADE ARRANGEMENT: Fixed 30° horizontal blades on 3/4 in. spacing.

DAMPER / FILTER / SCREEN: Bird screen standard; insect screen optional where scheduled.

FRAME: Fully extruded aluminum blades and wide flange frame.

FINISHES: White; Silver; Mill; Brown; Flat Black; Prime Coat; Custom

DATA: ANSI/ASHRAE Standard 70-2023.

SUGGESTED SPECIFICATION

Furnish and install Dayus Model DALTL-30-WF 30° Thinline Louver with Wide Flange of the type, finish, scheduled listed size/opening size, outside size, or inside size as applicable, and mounting method shown on the plans and air distribution schedules.

The 30° thinline louver with wide flange frame shall be constructed with fully extruded aluminum blades and wide flange frame and fixed 30° horizontal blades on 3/4 in. spacing.

The louver shall be supplied with bird screen standard; insect screen shall be optional where scheduled.

Units shall be furnished in Dayus [White; Silver; Mill; Brown; Flat Black; Prime Coat; Custom] finish as scheduled.

Provide scheduled options and related model variants as indicated.

The manufacturer shall provide current product submittal data and published performance information where applicable.

STANDARD FEATURES

- Wide flange for additional mounting surface coverage

- Fixed 30° horizontal blades on 3/4 in. spacing

- Hidden supports behind blades for clean appearance

- Bird screen standard; insect screen optional

- 1 in. standard dimensions; custom sizes to 1/16 in. increments

- Durable powder coat finish standard

RELATED OPTIONS / MODEL VARIANTS

- 30° or 45° blade angle as scheduled

- Core-only, flange frame, channel frame, or wide-flange variants where available

- Insect screen and all-aluminum options where scheduled

- Optional -Y blade variants where scheduled

Note: Confirm listed size/opening size, outside size, or inside size as applicable; finish; mounting method; damper/filter/screen requirements; and current Dayus model availability before issuing project specifications.