DALTLC-45

45° THINLINE LOUVER CORE

TYPE: 45° thinline louver core.

BLADE ARRANGEMENT: Fixed 45° horizontal blades on 3/4 in. spacing.

DAMPER / FILTER / SCREEN: Core only; bird screen not included unless scheduled with frame assembly.

FRAME: None.

FINISHES: White; Silver; Mill; Brown; Flat Black; Prime Coat; Custom

DATA: ANSI/ASHRAE Standard 70-2023.

SUGGESTED SPECIFICATION

Furnish and install Dayus Model DALTLC-45 45° Thinline Louver Core of the type, finish, scheduled listed size/opening size, outside size, or inside size as applicable, and mounting method shown on the plans and air distribution schedules.

The 45° thinline louver core shall be constructed with fully extruded aluminum blades and fixed 45° horizontal blades on 3/4 in. spacing.

The core-only louver shall not include bird screen unless scheduled with a frame assembly.

Units shall be furnished in Dayus [White; Silver; Mill; Brown; Flat Black; Prime Coat; Custom] finish as scheduled.

Provide scheduled options and related model variants as indicated.

The manufacturer shall provide current product submittal data and published performance information where applicable.

STANDARD FEATURES

- Fixed 45° horizontal blades on 3/4 in. spacing

- Hidden supports behind blades for clean appearance

- 1 in. standard dimensions; custom sizes to 1/16 in. increments

- Durable powder coat finish standard

RELATED OPTIONS / MODEL VARIANTS

- 30° or 45° blade angle as scheduled

- Core-only, flange frame, channel frame, or wide-flange variants where available

- Optional -Y blade variants where scheduled

Note: Confirm listed size/opening size, outside size, or inside size as applicable; finish; mounting method; damper/filter/screen requirements; and current Dayus model availability before issuing project specifications.