DARHED-45-WF

45° EXTRUDED RETURN AIR REGISTER WITH WIDE FLANGE

TYPE: Return register.

BLADE ARRANGEMENT: Fully extruded fixed blades at 45° on 3/4 in. spacing.

DAMPER / FILTER / SCREEN: Opposed blade damper.

FRAME: Wide-flange fully extruded aluminum frame.

FINISHES: W White; S Silver; M Mill; B Brown; FB Flat Black; Prime Coat; Special Order.

PERFORMANCE BASIS: ANSI/ASHRAE Standard 70-2023

SUGGESTED SPECIFICATION

Furnish and install Dayus Model DARHED-45-WF 45° Extruded Return Air Register with Wide Flange of the type, finish, scheduled listed size/opening size, outside size, or inside size as applicable, and mounting method shown on the plans and air distribution schedules.

The Return register shall be constructed with wide-flange fully extruded aluminum frame and fully extruded fixed blades at 45° on 3/4 in. spacing.

The unit shall include an opposed blade damper with concealed operator design.

Units shall be furnished in Dayus [W White; S Silver; M Mill; B Brown; FB Flat Black; Prime Coat; Special Order] finish as scheduled.

Provide scheduled options and related model variants as indicated.

The manufacturer shall provide published performance data for the return register, which shall be tested in accordance with ANSI/ASHRAE Standard 70-2023.

STANDARD FEATURES

- Wide flange for additional mounting surface coverage

- Fixed 45° deflection on 3/4 in. blade spacing

- Fully extruded architectural aluminum construction

- Hidden supports for clean appearance

- 2 in. standard dimensions; custom sizes to 1/16 in. increments

RELATED OPTIONS / MODEL VARIANTS

- No mounting / No screw-hole option

- Bird screen or insect screen where scheduled

- All aluminum construction

- Alternate 30° / 45° deflection models

- Opposed blade damper register variant where scheduled

Note: Confirm listed size/opening size, outside size, or inside size as applicable; finish; mounting method; damper/filter/screen requirements; and current Dayus model availability before issuing project specifications.