DAV

SINGLE DEFLECTION SUPPLY GRILLE - VERTICAL BLADE

MODEL DESCRIPTION: Single deflection aluminum grille with individually adjustable vertical blades for supply air.

TYPE: Supply grille.

BLADE ARRANGEMENT: One set of individually adjustable, semi-airfoil horizontal blades spaced on 2/3 in. centers

DAMPER: None.

FRAME: Standard architectural aluminum frame.

FINISHES: W - White; S - Silver; M – Mill

PERFORMANCE BASIS: ANSI/ASHRAE Standard 70-2023

SUGGESTED SPECIFICATION

Furnish and install Dayus Model DAV Single Deflection Supply Grilles of the type, finish, listed size/opening size, and mounting method shown on the plans and air distribution schedules. The grilles shall have one set of individually adjustable, semi-airfoil vertical blades spaced on 2/3 in. centers. The frame shall be architectural aluminum with roll-formed aluminum blades, pressure-fitted for rattle-free performance, and supplied with a sealing gasket to help prevent streaking on walls or ceilings. Units shall be furnished in Dayus W White, S Silver, or M Mill finish as scheduled.

(Optional) No screw-hole fastening and all-aluminum construction shall be provided where scheduled. For applications requiring volume control, provide the related Dayus opposed-blade or multi-shutter register model as scheduled.

The manufacturer shall provide published performance data for the grille or register, which shall be tested in accordance with ANSI/ASHRAE Standard 70-2023.

STANDARD FEATURES

- Individually adjustable, semi-airfoil vertical blades on 2/3 in. spacing

- Architectural aluminum construction with extruded frame and roll-formed blade.

- Pressure-fitted blades for rattle-free performance

- Sealing gasket to help prevent streaking on walls or ceilings

- Available on 2 in. standard dimensions; custom sizes manufactured up to 1/16 in. increments.

RELATED OPTIONS / MODEL VARIANTS

- DAVD - Opposed blade damper

- DA821 - Multi-shutter damper

- DAV-TB - No screw holes

- DAV-AA - All aluminum construction

- DAH - Horizontal blade version

Note: Use bracketed finish, size, mounting, and option selections from the project plans and air distribution schedule.